Operator Cover

Installation Instructions

The instructions contained below are supplemental to the instructions received with your door/operator.  Please review those instructions for any safety instructions, freight receiving instructions, etc prior to use of these instructions.

Note: Panels may vary in size and shape on your job compared to the below diagrams.

- After the door has been installed but BEFORE operator is installed, the Cover Mounting Plate must be installed.

- For smaller operators using the flat operator mounting plate, refer to Figure 1. For larger operators using an offset mounting plate, refer to Figure 2. If necessary, install bracing angles at this time with the operator mounting plate. Install the operator on the door, including the Cover Mounting Plate in the assembly.

Figure 1 – Flush Plate					Figure 2 – Offset Plate

- Finish installing drive chain and sprocket, and apply proper tension to chain by adjusting operator. Check door operation and adjust accordingly.

- On a flat surface, assemble cover by placing the End Panel with the flanges facing down. The top, bottom, front, and rear panels will look similar and their widths will correspond with the edge lengths of the end panel. Note that some panels may have cutouts for hand chains, cranks, or accessories.

- Locate the Bottom Panel and place its hem over the “bottom” flange on the end panel. (Figure 2)

- Locate the Front Panel and Rear Panel and do the same as with the bottom panel, ensuring the flanges on the front and rear panel overlap the bottom panel. (Figure 2)

- Locate the Top Panel and do the same, overlapping the front and rear panels. (Figure 2)

- Center and square the panels over the end panel and screw all panels to the end panel through the hem, roughly in the middle of the hem and 3/8” from the edge.

Figure 2 – Cover Assembly

- Screw the Top and Rear, and Rear and Bottom panels together at their overlapping edges, opposite of the end panel. (Figure 3).

Figure 3 – Cover Assembly

- Slide the assembled cover over the operator and cover mounting plate. (Figure 4)

- Butt the rear panel against the operator mounting plate so the cover is square and level, and screw the top panel into the cover mounting plate. (Figure 5)

Figure 4 – Cover Installation

- Checking again that the operator cover is square and level, screw the top and bottom panels to the cover mounting plate. Caulk all seams with commercially available sealant.

Figure 5 – Securing to Cover Mounting Plate